# QKD-Secured Site-to-Site VPN on AWS

Reference Architecture Guide
Palo Alto Networks VM-Series Firewalls with Quantum Key Distribution

# Overview

This document describes a Terraform-based reference architecture for deploying Quantum Key Distribution (QKD)-secured IPsec VPN tunnels using Palo Alto Networks VM-Series firewalls on Amazon Web Services (AWS).

The architecture deploys two security VPCs in the same AWS region, each containing a VM-Series firewall (PAN-OS 12.1+) with three network interfaces (management, untrust, and trust). QKD Key Management Entity (KME) devices connect to the management subnet at each site, providing quantum-derived encryption keys via the ETSI QKD 014 REST API. The IPsec tunnel runs between the untrust Elastic IPs.

# Architecture

Security VPC A (10.100.0.0/16)              Security VPC B (10.200.0.0/16)

  +--------------------+  IPsec Tunnel   +--------------------+
  | VM-Series FW A     |<==(QKD)========>| VM-Series FW B     |
  | eth0: mgmt (EIP)   |  untrust-to-   | eth0: mgmt (EIP)   |
  | eth1: untrust (EIP) |  untrust       | eth1: untrust (EIP) |
  | eth2: trust         |                | eth2: trust         |
  +--------+-----------+                 +--------+-----------+
           | mTLS (ETSI 014)                      | mTLS (ETSI 014)
  +--------v-----------+                 +--------v-----------+
  | QKD Device A       |<--quantum------>| QKD Device B       |
  | (KME) on mgmt      |  dark fiber    | (KME) on mgmt      |
  +--------------------+                 +--------------------+

## Components per Site

VM-Series Firewall: PAN-OS 12.1+ with 3 interfaces (mgmt, untrust, trust). Supports ETSI QKD 014 API for quantum key retrieval.

QKD KME Device: Quantum Key Management Entity (e.g., IDQuantique Clavis XG, Toshiba Q-KMS) connected to the management subnet. Provides quantum-derived symmetric keys via ETSI 014 REST API over mTLS.

Security VPC: Dedicated VPC with three subnets: management (with NAT Gateway), untrust (with Internet Gateway for IPsec EIPs), and trust (internal traffic).

# Network Design

## Subnet Layout

| Subnet | Site A CIDR | Site B CIDR | Purpose |
| --- | --- | --- | --- |
| mgmt | 10.100.0.0/24 | 10.200.0.0/24 | Management access + QKD KME API |
| untrust | 10.100.1.0/24 | 10.200.1.0/24 | Internet-facing, IPsec tunnel endpoints |
| trust | 10.100.2.0/24 | 10.200.2.0/24 | Internal / private traffic |

## Routing

| Subnet | Default Route | Purpose |
| --- | --- | --- |
| mgmt | NAT Gateway | Outbound for bootstrap S3, KME API, PAN-OS updates |
| untrust | Internet Gateway | EIP-based IPsec tunnel termination |
| trust | None (local only) | Internal traffic stays within VPC |

## Security Groups

| Group | Inbound Rules |
| --- | --- |
| mgmt | SSH (22), HTTPS (443) from allowed CIDRs; HTTPS (443) from mgmt subnet (KME API) |
| untrust | IKE (UDP 500), NAT-T (UDP 4500), ESP (protocol 50) for IPsec |
| trust | All traffic from VPC CIDR |

## VM-Series Interface Mapping

| Interface | ENI Index | Subnet | EIP | Purpose |
| --- | --- | --- | --- | --- |
| eth0 | 0 | mgmt | Yes | Management + KME API |
| eth1 | 1 | untrust | Yes | IPsec tunnel endpoint |
| eth2 | 2 | trust | No | Internal traffic |

# Traffic Flows

## IPsec Tunnel (QKD-Secured)

FW A untrust (EIP)  ---  UDP 500/4500 + ESP  --->  FW B untrust (EIP)

## QKD Key Fetch (ETSI 014 API)

Each firewall communicates with its local QKD KME device over the management interface using HTTPS with mutual TLS (mTLS) authentication.

FW A mgmt interface --> KME A (10.100.0.x:443) via ETSI QKD 014 API

FW B mgmt interface --> KME B (10.200.0.x:443) via ETSI QKD 014 API

## QKD Key Exchange (Physical Layer)

QKD devices exchange quantum keys over dedicated dark fiber:

Quantum channel: O-band fiber (~1310nm) for single-photon key exchange (BB84 protocol)

Classical channel: C-band fiber (~1550nm) for key reconciliation and privacy amplification

Note: QKD fiber range is typically 50-100 km depending on fiber loss and device model. Both sites must be within this range via dark fiber.

# Prerequisites

AWS account with permissions to create VPCs, EC2 instances, S3 buckets, IAM roles

Terraform >= 1.5

EC2 key pair in the target AWS region

VM-Series BYOL or Flex Credit license (PAN-OS 12.1+)

QKD devices (e.g., IDQuantique Clavis XG, Toshiba QKD) with dark fiber between sites

mTLS certificates for firewall-to-KME authentication

# Deployment Steps

## Step 1: Clone and Configure

cd aws-qkd
cp terraform.tfvars.example terraform.tfvars
# Edit terraform.tfvars with your values

## Step 2: Deploy Infrastructure

terraform init
terraform plan
terraform apply

## Step 3: Review Outputs

site_a_fw_mgmt_ip / site_b_fw_mgmt_ip — SSH/HTTPS to manage firewalls

site_a_fw_untrust_ip / site_b_fw_untrust_ip — IPsec tunnel endpoint IPs

site_a_mgmt_subnet_id / site_b_mgmt_subnet_id — Connect QKD KME devices here

## Step 4: Post-Deployment (Manual)

Connect QKD KME devices to each site's management subnet

Generate and install mTLS certificates on firewalls and KME devices

Configure QKD profiles on each VM-Series pointing to local KME ETSI 014 API

Configure IKE gateway with QKD profile and peer untrust EIP

Establish IPsec tunnel and verify with: show vpn ike-sa / show vpn ipsec-sa

# Terraform Modules Used

This project uses upstream modules from the Palo Alto Networks terraform-aws-swfw-modules repository (github.com/PaloAltoNetworks/terraform-aws-swfw-modules):

| Module | Purpose |
| --- | --- |
| vpc | VPC with security groups and Internet Gateway |
| subnet_set | Subnets with route tables |
| vmseries | VM-Series EC2 instance with ENIs |
| bootstrap | S3 bootstrap bucket + IAM role |
| nat_gateway_set | NAT Gateway for management outbound |

# Project File Structure

aws-qkd/
  versions.tf              - Terraform & provider versions
  variables.tf             - Input variables
  locals.tf                - Computed subnet CIDRs
  site_a.tf                - Site A deployment
  site_b.tf                - Site B deployment
  outputs.tf               - Tunnel endpoints, mgmt IPs
  terraform.tfvars.example - Example configuration
  modules/
    site/
      main.tf              - Module metadata
      variables.tf         - Site inputs
      outputs.tf           - Site outputs
      vpc.tf               - VPC, subnets, SGs, NAT GW, routes
      firewall.tf          - VM-Series instance (3 ENIs)
      bootstrap.tf         - S3 bootstrap + IAM
  docs/
    architecture.md        - Detailed design
    qkd_integration_guide.md - Post-deploy QKD setup